# DOCUMENTATION: STM32WB FAMILY COMPLETE IC COMPARISON

## 1. STM32WB FAMILY OVERVIEW

| Product Line | Core Architecture | Target Application | Flash Range | RAM Range |
| --- | --- | --- | --- | --- |
| STM32WBx5 | Dual-Core (M4 + M0+) | High performance, multi-protocol | 256KB - 1MB | 128KB - 256KB |
| STM32WBx0 Value Line | Dual-Core (M4 + M0+) | Cost-effective, balanced features | 320KB - 1MB | 48KB - 256KB |
| STM32WB0 Series | Single-Core (M0+) | Ultra-low power, cost-sensitive | 192KB - 512KB | 24KB - 64KB |
| STM32WB Modules | Dual-Core (M4 + M0+) | Pre-certified, easy integration | 320KB - 1MB | 48KB - 256KB |

The STM32WB family consists of over 40 different chip variants organized into 4 main product lines:

## 2. COMPLETE IC COMPARISON TABLE

### 2.1 STM32WBx5 Series (High Performance - Dual Core)

| Part Number | Flash | RAM | Package | GPIO | USB | I2C | SPI | UART | BLE | Zigbee |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STM32WB55CGU6 | 1MB | 256KB | UFQFPN-48 | 30 | ✅ | 2 | 1 | 2 | 5.4 | ✅ |
| STM32WB55CCU6 | 256KB | 128KB | UFQFPN-48 | 30 | ✅ | 2 | 1 | 2 | 5.4 | ✅ |
| STM32WB55CEU6 | 512KB | 256KB | UFQFPN-48 | 30 | ✅ | 2 | 1 | 2 | 5.4 | ✅ |
| STM32WB55RGV6 | 1MB | 256KB | VFQFPN-68 | 49 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB55REV6 | 512KB | 256KB | VFQFPN-68 | 49 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB55RCV6 | 256KB | 128KB | VFQFPN-68 | 49 | ✅ | 2 | 2 | 2 | 5.4 | ✅ |
| STM32WB55VCQ6 | 256KB | 128KB | UFBGA-129 | 72 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB55VEQ6 | 512KB | 256KB | UFBGA-129 | 72 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB55VGQ6 | 1MB | 256KB | UFBGA-129 | 72 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB55VGY6 | 1MB | 256KB | WLCSP-100 | 72 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB55VEY6 | 512KB | 256KB | WLCSP-100 | 72 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB55VCY6 | 256KB | 128KB | WLCSP-100 | 72 | ✅ | 2 | 2 | 2 | 5.3 | ✅ |
| STM32WB50CGU5 | 1MB | 128KB | UFQFPN-48 | 30 | ❌ | 1 | 1 | 1 | 5.4 | ✅ |
| STM32WB35CEU6 | 512KB | 96KB | UFQFPN-48 | 30 | ✅ | 2 | 1 | 2 | 5.4 | ✅ |
| STM32WB35CCU6 | 256KB | 96KB | UFQFPN-48 | 30 | ✅ | 2 | 1 | 2 | 5.4 | ✅ |
| STM32WB30CEU5 | 512KB | 96KB | UFQFPN-48 | 30 | ❌ | 1 | 1 | 1 | 5.4 | ✅ |

### 2.2 STM32WBx0 Value Line (Dual Core - Cost Optimized)

| Part Number | Flash | RAM | Package | GPIO | USB | I2C | SPI | UART | BLE | Zigbee |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STM32WB15CCU6 | 320KB | 48KB | UFQFPN-48 | 30 | ❌ | 1 | 1 | 1 | 5.3 | ❌ |
| STM32WB15CCY6 | 320KB | 48KB | WLCSP-49 | 25 | ❌ | 1 | 1 | 1 | 5.3 | ❌ |
| STM32WB10CCU5 | 320KB | 48KB | UFQFPN-48 | 30 | ❌ | 1 | 1 | 1 | 5.4 | ❌ |

### 2.3 STM32WB0 Series (New - Single Core M0+)

| Part Number | Core | Flash | RAM | Package | GPIO | I2C | SPI | UART | BLE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STM32WB09KEV6 | M0+ | 512KB | 16KB | VFQFPN-32 | 20 | 1 | 1 | 1 | 5.4 |
| STM32WB07CCV6 | M0+ | 256KB | 32KB | VFQFPN-48 | 32 | 1 | 2 | 1 | 5.4 |
| STM32WB05KZV6 | M0+ | 192KB | 24KB | VFQFPN-32 | 20 | 1 | 1 | 1 | 5.4 |
| STM32WB06CCV6 | M0+ | 256KB | 32KB | VFQFPN-48 | 32 | 1 | 2 | 1 | 5.4 |

### 2.4 STM32WB Modules (Pre-Certified)

| Module | Based On | Flash | RAM | Package | GPIO | USB | Certification | BLE | Price (INR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STM32WB1MMC | WB15CC | 320KB | 48KB | LGA-77 | 27 | ❌ | FCC, CE, IC, MIC | 5.4 | ₹558 – ₹744 |
| STM32WB5MMG | WB55CG | 1MB | 256KB | LGA-86 | 49 | ✅ | FCC, CE, IC, MIC | 5.4 + Zigbee | ₹930 – ₹1116 |

## 3. PART NUMBER DECODING

Understanding STM32WB part numbers helps you select the right chip:

| Position | Example | Meaning |
| --- | --- | --- |
| Family | STM32WB | Wireless MCU family |
| Series | 55, 35, 15, 05 | Performance level (higher = more features) |
| Memory | C, E, G | C=256KB, E=512KB, G=1MB Flash |
| Package | C, R, V, Y | C=48-pin, R=68-pin, V=100-pin, Y=WLCSP |
| Temperature | U, V | U=-40 to 85°C, V=-40 to 105°C |

Example: STM32WB55CGU6

- 55 = High performance series

- C = 48-pin package

- G = 1MB Flash

- U = -40 to 85°C

## 4. KEY FEATURE COMPARISON BY SERIES

### 4.1 Feature Availability Matrix

| Feature | WBx5 Series | WBx0 Value Line | WB0 Series | Modules |
| --- | --- | --- | --- | --- |
| Dual Core (M4+M0+) | Yes | Yes | No (M0+ only) | Yes |
| USB Peripheral | Yes (most) | No | No | Varies |
| LCD Driver | Yes (most) | No | No | Varies |
| Zigbee/Thread | Yes | No | No | Varies |
| 12-bit ADC | Yes | Yes | Yes | Yes |
| Quad-SPI | Yes | No | No | Varies |
| Crypto Engine | Yes | Yes | Yes | Yes |

### 4.2 Package Size Comparison

| Package | Size | Pin Count | Used In |
| --- | --- | --- | --- |
| UFQFPN-48 | 7x7mm | 48 | WB55CG, WB15CC, WB35, WB30 |
| VFQFPN-68 | 8x8mm | 68 | WB55RG, WB55RE |
| UFBGA-129 | 7x7mm | 129 | WB55VG, WB55VE |
| WLCSP-100 | 4.4x4.0mm | 100 | WB55VY, WB55VEY |
| VFQFPN-32 | 5x5mm | 32 | WB09, WB05 |
| LGA-77 | 6.5x10mm | 77 | WB1MMC (module) |
| LGA-86 | 7.3x11mm | 86 | WB5MMG (module) |

## 5. PRICE COMPARISON (1-unit pricing)

| Part Number | Flash | Key Feature |
| --- | --- | --- |
| STM32WB09KEV6 | 512KB | Lowest cost, single-core |
| STM32WB07CCV6 | 256KB | Good value, single-core |
| STM32WB30CEU5 | 512KB | Value line, dual-core |
| STM32WB15CCU6 | 320KB | USB + Zigbee |
| STM32WB35CCU6 | 256KB | USB + Zigbee |
| STM32WB55CGU6 | 1MB | Your dev board chip |
| STM32WB1MMC | 320KB | Pre-certified module |
| STM32WB5MMG | 1MB | Full module |

## 6. COMPLETE LIST OF ALL STM32WB VARIANTS

Based on STMicroelectronics official documentation and distributor data:

### Modules (2 variants) (Ready-to-use, no RF design)

- STM32WB1MMC, STM32WB5MMG

### WBx5 Series (16+ variants) (High performance, full wireless support)

- STM32WB55CG, STM32WB55CC, STM32WB55CE

- STM32WB55RG, STM32WB55RC, STM32WB55RE

- STM32WB55VG, STM32WB55VC, STM32WB55VE

- STM32WB55VY, STM32WB55VEY, STM32WB55VCY

- STM32WB50CG

- STM32WB35CE, STM32WB35CC

- STM32WB30CE

### WBx0 Value Line (3+ variants) (Low cost, BLE only)

- STM32WB15CC, STM32WB15CCY

- STM32WB10CC

### WB0 Series (11+ variants) (Ultra low cost, basic IoT)

- STM32WB09KE, STM32WB09TE

- STM32WB07CC, STM32WB07KC

- STM32WB06CC, STM32WB06KC

- STM32WB05KZ, STM32WB05TZ

TOTAL: 32+ distinct STM32WB variants

## 7. ADC QUALITY ACROSS ALL STM32WB BOARDS

All STM32WB series chips share the same high-quality 12-bit ADC. There is no difference in ADC quality between the WB55, WB15, or WB0 series.

### 7.1 ADC Specifications (Common to All STM32WB Chips)

| ADC Feature | Specification | Why It Matters for Your Project |
| --- | --- | --- |
| Resolution | 12-bit native | Reads 0-4095 values - excellent for sensors |
| Max with Oversampling | Up to 16-bit | Software averaging for noise reduction |
| Max Conversion Rate | 4.26 Msps | Very fast - reads sensor instantly |
| Min Sampling Time | 39 ns | Can capture fast-changing signals |
| External Channels | Up to 16 | Connect multiple sensors |
| Internal Channels | 3 (Vref, temperature, battery) | Monitor internal health |
| Low Power Mode | 200 µA/Msps | Battery friendly |
| Single-ended Mode | Yes | Simple sensor connection |
| Differential Mode | Yes | Better noise rejection |
| Hardware Oversampling | Yes | Built-in averaging |

## 8. CERTIFICATIONS ACROSS ALL STM32WB BOARDS

All STM32WB chips share the same certifications at the silicon level.

### 8.1 Common Certifications

| Certification | Region | Status | What It Means for Your Product |
| --- | --- | --- | --- |
| ETSI EN 300 328 | Europe | Compliant | Legal to sell in Europe |
| EN 300 440 | Europe | Compliant | Short-range radio legal |
| FCC CFR47 Part 15 | USA | Compliant | Legal to sell in USA |
| ARIB STD-T66 | Japan | Compliant | Legal to sell in Japan |
| RoHS 2011/65/EU | Europe | Compliant | No hazardous materials |
| RoHS 2015/863 | Europe | Compliant | Extended chemical ban |
| REACH | Europe | Unaffected | Safe chemicals |
| Bluetooth SIG | Global | Certified (BLE 5.2/5.3/5.4) | Official Bluetooth logo allowed |

### 8.2 Certification by Series

| Certification | WBx5 Series | WBx0 Series | WB0 Series |
| --- | --- | --- | --- |
| RF (ETSI, FCC, ARIB) | Yes | Yes | Yes |
| RoHS / REACH | Yes | Yes | Yes |
| Bluetooth SIG | BLE 5.3/5.4 | BLE 5.2 | BLE 5.4 |
| Zigbee / Thread | Yes | No | No |

## 9. RECOMMENDATION FOR NEW DESIGN

### 9.1 Is the STM32WB Family Recommended for New Designs?

YES - The STM32WB family is recommended for new designs.

STMicroelectronics has committed to a 10-year longevity plan for the STM32WB series, ensuring long-term availability for your product. The family remains Active (in volume production) with ongoing support

### 9.2 Which Chips to Choose (Recommended)

| Recommendation | Chip / Series | Why |
| --- | --- | --- |
| BEST CHOICE | STM32WB55 Series (WB55CEU6, WB55CGU6) | Full features, dual-core, USB, Zigbee, most mature, best documentation |
| GOOD CHOICE | STM32WB15 Series (WB15CCU6 - Rev Z) | Lower cost, dual-core, no USB (good if you removed USB-C port) |
| EASIEST CHOICE | STM32WB1MMC Module | Pre-certified, no RF design needed, saves 6-12 months development time |

## 10. FINAL SELECTION OF BOARD & CHIP

Based on your complete project requirements (ADC reading + BLE parameter updates + GUI communication via UART header + Headers-only + Compact design + Battery-powered), here is the final selection with proper reasoning.

## 10.1 Final Chip Selection – Complete Specifications

| Parameter | Value |
| --- | --- |
| Part Number | STM32WB55CEU6 |
| Series | STM32WBx5 (High Performance) |
| Package | UFQFPN-48 (7mm x 7mm) |
| Core | Dual-Core: Cortex-M4 (64 MHz) + Cortex-M0+ (32 MHz) |
| Flash | 512 KB |
| RAM | 256 KB |
| Operating Voltage | 1.71V – 3.6V |
| Temperature Range | -40°C to +85°C |
| Status for New Design | FULLY RECOMMENDED |

### Wireless (BLE)

| Parameter | Value |
| --- | --- |
| BLE Version | 5.4 |
| Bluetooth SIG Certification | ✅ Certified |
| Multi-Protocol | BLE + Zigbee + Thread (IEEE 802.15.4) |
| RX Sensitivity | -96 dBm (BLE 1Mbps) |
| TX Power | Up to +6 dBm (1 dB steps) |
| Data Rate | Supports 2 Mbps |
| Integrated Balun | ✅ Yes |

### Communication Interfaces

| Interface | Count | Pins Available | Purpose for Your Project |
| --- | --- | --- | --- |
| USART | 1 | PA9 (TX), PA10 (RX) | GUI Communication via UART Header |
| LPUART | 1 | PA2 (TX), PA3 (RX) | Low-power wake-up from BLE |
| I2C | 2 | PB6 (SCL), PB7 (SDA) – I2C1 PB10 (SCL), PB11 (SDA) – I2C2 | External sensors (optional) |
| SPI | 2 | PA5, PA6, PA7 – SPI1 PB13, PB14, PB15 – SPI2 | High-speed devices (optional) |
| USB 2.0 FS | 1 | PA11 (D+), PA12 (D-) | Optional – you are NOT using (headers-only) |
| SWD | 1 | PA13 (SWDIO), PA14 (SWCLK) | Debugging & Programming |
| GPIO | 30 | Various | General purpose I/O |

### ADC (Analog to Digital Converter)

| Parameter | Value |
| --- | --- |
| Resolution | 12-bit native |
| Max with Oversampling | Up to 16-bit |
| Max Conversion Rate | 4.26 Msps |
| Number of Channels | Up to 16 external channels |
| Internal Channels | 3 (Vref, temperature sensor, battery) |
| Low Power Mode | 200 µA/Msps |
| Single-ended Mode | Yes |
| Differential Mode | Yes |

### Power Consumption

| Mode | Current | When to Use |
| --- | --- | --- |
| Shutdown | 13 nA | Long-term storage |
| Standby (with RTC) | 600 nA | Deep sleep, wake on timer |
| Stop | 2.1 µA | Light sleep, fast wake |
| Active (CPU) | 53 µA/MHz | Processing (ADC, GUI logic) |
| BLE Receive | 4.5 mA | Listening for commands |
| BLE Transmit @0dBm | 5.2 mA | Sending data |

### Security Features

| Feature | Description |
| --- | --- |
| AES Engine | 256-bit hardware encryption |
| PKA | Public Key Accelerator |
| RNG | True Random Number Generator |
| PCROP | Proprietary Code Readout Protection |

### Certifications

| Certification | Region | Status |
| --- | --- | --- |
| ETSI EN 300 328 | Europe | Compliant |
| EN 300 440 | Europe | Compliant |
| FCC CFR47 Part 15 | USA | Compliant |
| ARIB STD-T66 | Japan | Compliant |
| RoHS 2011/65/EU | Europe | Compliant |
| REACH | Europe | Compliant |
| Bluetooth SIG | Global | Certified (BLE 5.4) |

### Pin Summary for Your Headers-Only Design

| Header | Pin | STM32 Pin | Function | Connect To |
| --- | --- | --- | --- | --- |
| SWD (4-pin) | 1 | PA13 | SWDIO | ST-Link SWDIO |
|  | 2 | PA14 | SWCLK | ST-Link SWCLK |
|  | 3 | NRST | Reset | ST-Link NRST |
|  | 4 | GND | Ground | ST-Link GND |
| UART (3-pin) | 1 | PA9 | USART1_TX | USB-to-Serial RX |
|  | 2 | PA10 | USART1_RX | USB-to-Serial TX |
|  | 3 | GND | Ground | USB-to-Serial GND |
| Power (2-pin) | 1 | VDD (3.3V) | Power input | 3.3V Regulator |
|  | 2 | GND | Ground | Ground |
| ADC (1-pin) | 1 | PA0 (ADC1_IN0) | Analog Input | Sensor Output |
| I2C (Optional) | 1 | PB6 | I2C1_SCL | External Sensor SCL |
|  | 2 | PB7 | I2C1_SDA | External Sensor SDA |
|  | 3 | GND | Ground | Ground |

### Summary

| Category | Specification |
| --- | --- |
| Chip | STM32WB55CEU6 |
| BLE Version | 5.4 |
| ADC | 12-bit, 4.26 Msps, 16 channels |
| GUI Interface | UART (PA9/TX, PA10/RX) |
| Debug Interface | SWD (PA13, PA14, NRST) |
| I2C Available | 2 channels (PB6/PB7, PB10/PB11) |
| UART Available | 1x USART + 1x LPUART |
| GPIO Total | 30 |
| Standby Power | 600 nA |
| Active Power | 53 µA/MHz |
| Package | UFQFPN-48 (7x7mm) |
| New Design | RECOMMENDED |

### 10.2 Point Reasons for Selection

|  | Reason | Explanation |
| --- | --- | --- |
| 1 | Dual-Core Architecture | BLE stack runs on dedicated M0+ core. Your ADC reading and GUI communication run on M4 core. No interference between wireless and application tasks. |
| 2 | 12-bit ADC (4.26 Msps) | Excellent for sensor reading. Hardware oversampling up to 16-bit. Same high-quality ADC across all STM32WB series. |
| 3 | BLE 5.4 Certified | Latest Bluetooth standard. Supports parameter updates over BLE. Bluetooth SIG certified. |
| 4 | Perfect Memory Size | 512KB Flash / 256KB RAM. Not too little (like WB15's 48KB RAM) and not wasteful overkill (like WB55's 1MB Flash). Perfect balance for your needs. |
| 5 | UART for GUI Communication | USART1 on PA9 (TX) and PA10 (RX) – perfect for connecting USB-to-Serial converter for GUI communication. |
| 6 | SWD for Debugging | PA13 (SWDIO) and PA14 (SWCLK) – standard debug interface. Works with ST-Link V2/V3. |
| 7 | Ultra-Low Power | 600 nA standby, 13 nA shutdown, 53 µA/MHz active. Excellent for battery-powered products. |
| 8 | Mature & Stable | Released in 2019, all bugs fixed, no major errata. Safe for production. |
| 9 | 10-Year Longevity | ST guarantees availability until at least 2036. Future-proof for your product. |
| 10 | All Certifications Included | FCC, CE, ARIB, RoHS, REACH, Bluetooth SIG. Saves certification cost and time. |

### 10.3 Why NOT Other Chips

| Chip | Why NOT Selected |
| --- | --- |
| STM32WB55CGU6 (1MB Flash) | Too expensive,too much memory – overkill for your needs |
| STM32WB15CCU6 | Less memory (320KB Flash / 48KB RAM) – tight for future; has errata on Revision B |
| STM32WB09KE (WB0 Series) | Single-core – BLE and your code share one processor; NOT recommended |
| nRF52840 (Nordic) | More expensive, new SDK to learn |
| ESP32-C3 | Higher power consumption, has WiFi you don't need |